Homework 5

Fall 2014 DSD

Name Ivan E Garcia	 	abc123 eaa108			Date 9/23/2026

1.  Design a 4 bit ripple carry adder and use gate delay of 5 ns calculate the delay for a 4 bit add

`timescale 1ns / 1ps

module RippleCarry4bitadder(

input wire [3:0] a,

input wire [3:0] b,

input wire CarryIn,

output wire CarryOut,

output wire [3:0] sum

);

wire c1, c2, c3;

FullAdder FA0 (.A(a[0]), .B(b[0]), .Cin(CarryIn), .Sum(sum[0]), .Cout(c1));

FullAdder FA1 (.A(a[1]), .B(b[1]), .Cin(c1), .Sum(sum[1]), .Cout(c2));

FullAdder FA2 (.A(a[2]), .B(b[2]), .Cin(c2), .Sum(sum[2]), .Cout(c3));

FullAdder FA3 (.A(a[3]), .B(b[3]), .Cin(c3), .Sum(sum[3]), .Cout(CarryOut));

endmodule

module FullAdder(

input wire A, B, Cin,

output wire Cout, Sum

);

wire gate_1_out;

wire gate_2_out;

wire gate_3_out;

xor #5 xor_gate1 (gate_1_out, A, B);

xor #5 xor_gate2 (Sum, gate_1_out, Cin);

and #5 and_gate3 (gate_2_out, gate_1_out, Cin);

and #5 and_gate4 (gate_3_out, A, B);

or #5 or_gate5 (Cout, gate_2_out, gate_3_out);

Endmodule

2.  Design a 4 bit carry look ahead adder and use gate delay of 5ns and calculate the delay for a 4 bit add

`timescale 1ns / 1ps

module CarryLookAhead4BitAdder(

input wire [3:0] A,

input wire [3:0] B,

input wire Cin,

output wire [3:0] Sum,

output wire Cout

);

wire [3:0] G;

wire [3:0] P;

wire [3:0] C;

wire c1_term;

wire c2_term1;

wire c2_term2;

wire c3_term1;

wire c3_term2;

wire c3_term3;

wire c4_term1;

wire c4_term2;

wire c4_term3;

wire c4_term4;

// Starting carry

assign C[0] = Cin;

// Generate and propagate signals

// These four-bit operations represent four parallel gates.

assign #5 G = A & B;

assign #5 P = A ^ B;

// Calculate C1

assign #5 c1_term = P[0] & Cin;

assign #5 C[1] = G[0] | c1_term;

// Calculate C2

assign #5 c2_term1 = P[1] & G[0];

assign #5 c2_term2 = P[1] & P[0] & Cin;

assign #5 C[2] = G[1]

| c2_term1

| c2_term2;

// Calculate C3

assign #5 c3_term1 = P[2] & G[1];

assign #5 c3_term2 = P[2] & P[1] & G[0];

assign #5 c3_term3 = P[2] & P[1] & P[0] & Cin;

assign #5 C[3] = G[2]

| c3_term1

| c3_term2

| c3_term3;

// Calculate final carry-out C4

assign #5 c4_term1 = P[3] & G[2];

assign #5 c4_term2 = P[3] & P[2] & G[1];

assign #5 c4_term3 = P[3] & P[2] & P[1] & G[0];

assign #5 c4_term4 = P[3] & P[2] & P[1] & P[0] & Cin;

assign #5 Cout = G[3]

| c4_term1

| c4_term2

| c4_term3

| c4_term4;

// Calculate all four sum bits

assign #5 Sum = P ^ C;

endmodule

3.  Does the delay difference between the designs increase linearly or not as the number of bits increases?

Since the carry must pass through every bit, the ripple-carry adder’s delay increases linearly.  The

carry look-ahead adder makes carries parallel, so its delay grows far slower. Therefore,

the delay difference between the two designs becomes larger(mostly linear) as bits increase.

The ripple-carry adder’s delay increases linearly because the carry must pass through every bit.