DSD F14

Homework #4

- Write a Verilog model for a 74HC192 synchronous 4-bit up/down counter.  Don’t include any timing data.   Make sure you include in the sensitivity list (Down, Up, Clr, Load)

module HC192 (

input        Up,

input        Down,

input        Clr,

input        Load,

input  [3:0] D,

output reg [3:0] Q

);

always @(posedge Up or posedge Down or negedge Clr or negedge Load)

begin

if (!Clr)

Q <= 4'b0000;

else if (!Load)

Q <= D;

else if (Up)

Q <= Q + 1'b1;

else if (Down)

Q <= Q - 1'b1;

end

endmodule

- Write Verilog code for a module describing a 8-bit shift register that shifts bits in from input SI and out an output SO.  Also include control signals Clk and EN (and enable active Hi).  Thus:  You should have 3 inputs SI, Clk, EN and 1 out SO:  all these are single bit signals.  The shifting causes 1 bit to go in and 1 bit to go out and remembers the other 7 bits, but the bit just shifted in will not get shifted out for 8 more clock edges.  Use a rising edge clock.

- Write a Verilog module for a SR Latch holding 1 bit.  Do it 2 ways:

  - Use an Boolean equation

  - Use a gate level description